TOOELE CITY BUILDING CODES AND DESIGN CRITERIA

BUILDING CODES:

- 2024 INTERNATIONAL BUILDING CODE (IBC) INCLUDING APPENDICES C AND J.

- 2021 INTERNATIONAL RESIDENTIAL CODE (IRC) INCLUDING APPENDICES AQ AND AE

- 2024 INTERNATIONAL PLUMBING CODE (IPC).

- 2024 INTERNATIONAL MECHANICAL CODE (IMC).

- 2024 INTERNATIONAL FUEL GAS CODE (IFC).

- 2024 INTERNATIONAL EXISTING BUILDING CODE (IEBC).

- 2023 INTERNATIONAL ENERGY CONSERVATION CODE (IECC) USE COMMERCIAL PROVISIONS

- 2015 INTERNATIONAL ENERGY CONSERVATION CODE (IECC) USE RESIDENTIAL PROVISIONS.

- 2024 INTERNATIONAL EXISTING BUILDING CODE (IEBC).

- 2023 NATIONAL ELECTRICAL CODE (NEC).

- ICC / ANSI A117.1-2009.

- UTAH STATE AMENDMENTS AS PER TITLE 15A STATE CONSTRUCTION AND FIRE CODES ACT.

DESIGN CRITERIA:

- SNOW LOAD:

GROUND SNOW LOAD USE UTAH STATE UNIVERSITY

https://www.usu.edu/utahsnowload/

ROOF LOAD DETERMINED PER CHAPTER 7 OF ASCE 7-16.

- WIND:

IRC SECTION:  	105 MPH 3 SECOND GUST EXPOSURE C.

IBC SECTION: 	RISK CATEGORY I 95

RISK CATEGORY II 100

RISK CATEGORY III 105 (MOST COMMON)

RISK CATEGORY IV 110

- SESMIC ZONE:

IRC - D1

IBC - D

- FROST DEPTH:

30 INCHES.

- SOIL:

IRC ASSUMED 1500 PSF

IBC SITE SPECIFIC GEOTECHNICAL REPORT

- CLIMATE ZONE:

5B

- WEATHERERING:

SEVERE